# Automated MoClo assembly protocol

Hello! This document contains a protocol for the assembly of genetic parts using MoClo assembly with an automated liquid handler. This document was produced by the software 'SynBioMate' (https://github.com/phoenixgater/SynBioMate)

The protocols and fusion sites in this protocol are designed to be compatible with the EcoFlex MoClo kit, available from: http://www.addgene.org/kits/freemont-ecoflex-moclo/

Notes on this document and its contents:
-For restriction sites detected in open reading frames (coding regions, CDSs, ORFs), a codon in this region has been swapped to ensure that the part is MoClo compatible. This codon swap is specified in this documents appendix.

-If an ATG start codon could not be found at the start of an ORF/CDS part, this has been added, and noted in the appendix as well. It is, however, highly advised that the use of these parts be reconsidered

-This software is unable to suggest base substitutions for excluded restriction sites detected in non-coding genetic parts (e.g Promoters, RBSs, etc), but the presence of these restriction sites will be noted in this documents appendix. It is highly advised that the use of these parts be reconsidered

Please note that the contents of this document will only be valid for the user-input parameters that were given at the time of this documents creation. These were:
Signal peptide included: No
Chassis system: E. coli
Transcription unit quantity: 2
Assembly method: Automatic

# Protocol

## Creating level 0 library and cloning into level 0 plasmid backbones

a) For each of the BioParts in this documents appendix (all parts EXCLUDING CDS/ORF): 
1. Design and create/purchase forward and reverse primers for the BioPart
2. Anneal forward and reverse primers (20 uM of each) in 1 x T4 DNA ligase buffer at 90°C for 1 min, followed by cooling on ice
3. Phosphorylate with T4 PNK for 1 hour at 37°C
4. Digest 40 ng/uL-1 (24nM) of pBP-lacZa with 1 unit of NdeI and SphI in CutSmart buffer for 1 hour at 37°C, heat inactivate at 80°C for 10 min
5. Add 2 uL (80 ng) of NdeI-SphI digested pBP-lacZa plasmid with 2 uL of 200 nM annealed primers in 2 x Rapid ligation buffer (Promega) and 1-3 units of T4 ligase (Promega)
6. Incubate at room-temperature (19-21 °C) for 30-60 min
7. Transform 5 uL into 50 uL of DH10a competent cells
8. Grow on 35 ug mL-1 chloramphenicol LB plates with 0.1 mM IPTG and 40 ug mL-1 X-Gal (Sigma) for 24 hrs
9. Place in cold room overnight for clear visualisation of negative blue colonies
10. Pick 1-2 white colonies for plasmid preparation and sequencing

b) For each of the CDS/ORF parts in this documents appendix: 
1. Design and create/purchase forward and reverse primers for the CDS part
Anneal forward and reverse primers (20 uM of each) in 1 x T4 DNA ligase buffer at 90°C for 1 min, followed by cooling on ice
3. Phosphorylate with T4 PNK for 1 hour at 37°C
4. Digest 40 ng/uL-1 (24nM) of pBP-ORF with 1 unit of NdeI and BamHI in CutSmart buffer for 1 hour at 37°C, heat inactivate at 80°C for 10 min
5. Add 2 uL (80 ng) of NdeI-BamHI digested pBP-ORF plasmid with 2 uL of 200 nM annealed primers in 2 x Rapid ligation buffer (Promega) and 1-3 units of T4 ligase (Promega)
6. Incubate at room-temperature (19-21 °C) for 30-60 min
7. Transform 5 uL into 50 uL of DH10a competent cells
8. Grow on 35 ug mL-1 chloramphenicol LB plates with 0.1 mM IPTG and 40 ug mL-1 X-Gal (Sigma) for 24 hrs
9. Place in cold room overnight for clear visualisation of negative blue colonies
10. Pick 1-2 white colonies for plasmid preparation and sequencing

## Creating level 1 transcription units (TUs)

Plates required:
-2x Echo® Qualified 384-Well Polypropylene Source Microplate (384PP) (One of these is used as an output plate and must remain empty)
-Echo® Qualified 384-Well COC Source Microplate, Low Dead Volume (384LDV)
-Echo® Qualified Reservoir (6RES)

a) Add genetic parts and level 1 transcription unit backbones into their corresponding wells in the Echo® Qualified 384-Well Polypropylene Source Microplate (384PP) as specified in the table below

| Well | Genetic part | Quantity (nl) |
| --- | --- | --- |
| A1 | BBa_J23119 | 17000 |
| A2 | BBa_B0031 | 17000 |
| A3 | BBa_K1323010 | 17000 |
| A4 | BBa_B1001 | 17000 |
| A5 | pTU1-A-lacZ | 16000 |
| A6 | pTU1-B-lacZ | 16000 |

b) Add deionised water to the corresponding well(s) in the Echo® Qualified reservoir (6RES) as specified in the table below:

| Well | Reagent | Quantity (nl) |
| --- | --- | --- |
| A1 | deionised water | 264750 |

c) Add reagents to their corresponding wells in the Low Dead Volume Echo® Qualified 384-Well COC Source Microplate (384LDV) as specified in the table below:

| Well | Reagent | Quantity (nl) |
| --- | --- | --- |
| A1 | 10x DNA ligase buffer (Promega) | 6000 |
| A2 | 1-3 units T4 DNA ligase (Promega) | 6750 |
| A3 | BsaI-HF (NEB) | 7500 |

Locations of produced transcription unit variants in output plate:

| Well | contained TU variant | volume (nl) |
| --- | --- | --- |
| A1 | Transcription unit 1 variant 1 reaction 1:1 | 5000 |
| A2 | Transcription unit 1 variant 1 reaction 1:2 | 5000 |
| A3 | Transcription unit 1 variant 1 reaction 2:1 | 5000 |
| A4 | Transcription unit 2 variant 1 reaction 1:1 | 5000 |
| A5 | Transcription unit 2 variant 1 reaction 1:2 | 5000 |
| A6 | Transcription unit 2 variant 1 reaction 2:1 | 5000 |

d) Run a PCR protocol for this output plate, consisting of:
15-30 cycles of:
15-30 cycles of:
-5 minutes at 37°C
-10 minutes at 16°C
Followed by (only once):
-5 minutes at 50°C
-5 minutes at 80°C 
e) Transform 5 uL of each output mixture into 50uL of chemically compotent Escherichia coli (E. coli) dH10a by heat shock transformation (One mixture per cell culture)

## Creating level 2 transcription units (TUs)

Plates required:
-2x Echo® Qualified 384-Well Polypropylene Source Microplate (384PP) (One of these is used as an output plate and must remain empty)
-Echo® Qualified 384-Well COC Source Microplate, Low Dead Volume (384LDV)
-Echo® Qualified Reservoir (6RES)
a) Recover and re-suspend level 1 transcription units from transformed cells in previous step
b) Add level 1 transcription units and level 2 plasmid backbones to their specified wells in the Echo® Qualified 384-Well Polypropylene Source Microplate (384PP) as specified in the table below:

| Well | Genetic part | Quantity (nl) |
| --- | --- | --- |
| A1 | Transcription unit 1 variant 1 | 16000 |
| A2 | Transcription unit 2 variant 1 | 16000 |
| A3 | pTU2-a-RFP | 16000 |

b) Add deionised water to the corresponding well(s) in the Echo® Qualified reservoir (6RES) as specified in the table below:

| Well | Reagent | Quantity (nl) |
| --- | --- | --- |
| A1 | deionised water | 259375 |

c) Add reagents to their corresponding wells in the Low Dead Volume Echo® Qualified 384-Well COC Source Microplate (384LDV) as specified in the table below:

| Well | Reagent | Quantity (nl) |
| --- | --- | --- |
| A1 | 10x DNA ligase buffer (Promega) | 4500 |
| A2 | 1-3 units T4 DNA ligase (Promega) | 6375 |
| A3 | BsmBI (NEB) | 6750 |

Locations of produced transcription unit variants in output plate:

| Well | contained TU variant | volume (nl) |
| --- | --- | --- |
| A1 | Level 2 construct variant 1 reaction 1:1 | 5000 |
| A2 | Level 2 construct variant 1 reaction 1:2 | 5000 |
| A3 | Level 2 construct variant 1 reaction 2:1 | 5000 |

# Appendix

## Promoters

### Modified BBa_J23119

Description: constitutive promoter family member

Modifications:
-Restriction site cctagg (BamHI) detected at position 31-36
-Prefix added for NdeI overhang (ta), BsaI restriction site (ggtctca) and golden gate promoter fusion site (ctat)
-Suffix added for golden gate promoter fusion site (gtac), BsaI restriction site (agagacc), and SphI overhang (catg)

Part design: 
5' taggtctcactatttgacagctagctcagtcctaggtataatgctagcgtacagagacccatg 3'
3'     ccagagtgataaactgtcgatcgagtcaggatccatattacgatcgcatgtctctgg         5'

## Ribosome binding sites (RBSs)

### Modified BBa_B0031

Description: RBS.2 (weak) -- derivative of BBa_0030

Modifications:
-Prefix added for NdeI overhang (ta), BsaI restriction site (ggtctca) and golden gate RBS fusion site (gtac)
-Suffix added for golden gate RBSfusion site (cata), BsaI restriction site (agagacc), and SphI overhang (catg)

Part design: 
5' taggtctcagtactcacacaggaaacccataagagacccatg 3'
3'     ccagagtcatgagtgtgtcctttgggtattctctgg         5'

## Coding regions (CDSs)

### Modified BBa_K1323010

Description: YFP Coding Sequence

Modifications:
-Prefix added for NdeI overhang (t)
-Suffix added for BamHI overhang (ctag)

Part design: 
5' tatggtttcaaaaggtgaagaattatttacaggtgttgttccaattttggttgaattagatggtgatgttaatggtcataaattttcagtttcaggtgaaggtgaaggtgatgcaacttatggtaaattaacattaaaatttatttgtacaacaggtaaattaccagttccatggccaacattagttacaacatttggttatggtttacaatgttttgcacgttatccagatcacatgaaacaacatgatttttttaaatcagcaatgcctgaaggttatgttcaagaacgtacaattttttttaaagatgatggcaactataaaacacgtgctgaagttaaatttgaaggtgatacattagttaatcgtattgaattaaaaggtattgattttaaagaagatggtaatattttaggtcataaattagaatataattataactcacataatgtttatatcatggcagataaacaaaaaaatggtattaaagttaattttaaaattcgtcataatattgaagatggttcagttcaattagcagatcattatcaacaaaatacacctattggtgatggtccagttttattaccagataatcattatttatcatatcaatcagcattatcaaaagatccaaatgaaaaacgtgatcacatggttttattagaatttgttacagcagcaggtattacattaggtatggatgaattatataaataactag 3'
3'     accaaagttttccacttcttaataaatgtccacaacaaggttaaaaccaacttaatctaccactacaattaccagtatttaaaagtcaaagtccacttccacttccactacgttgaataccatttaattgtaattttaaataaacatgttgtccatttaatggtcaaggtaccggttgtaatcaatgttgtaaaccaataccaaatgttacaaaacgtgcaataggtctagtgtactttgttgtactaaaaaaatttagtcgttacggacttccaatacaagttcttgcatgttaaaaaaaatttctactaccgttgatattttgtgcacgacttcaatttaaacttccactatgtaatcaattagcataacttaattttccataactaaaatttcttctaccattataaaatccagtatttaatcttatattaatattgagtgtattacaaatatagtaccgtctatttgtttttttaccataatttcaattaaaattttaagcagtattataacttctaccaagtcaagttaatcgtctagtaatagttgttttatgtggataaccactaccaggtcaaaataatggtctattagtaataaatagtatagttagtcgtaatagttttctaggtttactttttgcactagtgtaccaaaataatcttaaacaatgtcgtcgtccataatgtaatccatacctacttaatatatttatt         5'

## Terminators

### Modified BBa_B1001

Description: Terminator (artifical, small, %T~=90)

Modifications:
-Prefix added for NdeI overhang (ta), BsaI restriction site (ggtctca) and golden gate terminator fusion site (tcga)
-Suffix added for golden gate terminator fusion site (tgtt), BsaI restriction site (agagacc), and SphI overhang (catg)

Part design: 
5' taggtctcatcgaaaaaaaaaaccccgcttcggcggggttttttttttgttagagccccatg 3'
3'     ccagagtagcttttttttttggggcgaagccgccccaaaaaaaaaacaatctcggg         5'

## Level 1 transcription units

### Transcription unit 1 variant 1

Parts: Modified BBa_J23119, Modified BBa_B0031, Modified BBa_K1323010, Modified BBa_B1001,

Notes: Prefix atct and suffix tgcc fusion sites conferred to sequence by level 1 plasmid backbone pTU1-A-lacZ

Sequence (Excluding plasmid backbone): 5' atctctatttgacagctagctcagtcctaggtataatgctagcgtactcacacaggaaacccatatggtttcaaaaggtgaagaattatttacaggtgttgttccaattttggttgaattagatggtgatgttaatggtcataaattttcagtttcaggtgaaggtgaaggtgatgcaacttatggtaaattaacattaaaatttatttgtacaacaggtaaattaccagttccatggccaacattagttacaacatttggttatggtttacaatgttttgcacgttatccagatcacatgaaacaacatgatttttttaaatcagcaatgcctgaaggttatgttcaagaacgtacaattttttttaaagatgatggcaactataaaacacgtgctgaagttaaatttgaaggtgatacattagttaatcgtattgaattaaaaggtattgattttaaagaagatggtaatattttaggtcataaattagaatataattataactcacataatgtttatatcatggcagataaacaaaaaaatggtattaaagttaattttaaaattcgtcataatattgaagatggttcagttcaattagcagatcattatcaacaaaatacacctattggtgatggtccagttttattaccagataatcattatttatcatatcaatcagcattatcaaaagatccaaatgaaaaacgtgatcacatggttttattagaatttgttacagcagcaggtattacattaggtatggatgaattatataaataatcgaaaaaaaaaaccccgcttcggcggggttttttttttgtttgcc 3'

### Transcription unit 2 variant 1

Parts: Modified BBa_J23119, Modified BBa_B0031, Modified BBa_K1323010, Modified BBa_B1001,

Notes: Prefix tgcc and suffix ccgg fusion sites conferred to sequence by level 1 plasmid backbone pTU1-B-LacZ

Sequence (Excluding plasmid backbone): 5' tgccctatttgacagctagctcagtcctaggtataatgctagcgtactcacacaggaaacccatatggtttcaaaaggtgaagaattatttacaggtgttgttccaattttggttgaattagatggtgatgttaatggtcataaattttcagtttcaggtgaaggtgaaggtgatgcaacttatggtaaattaacattaaaatttatttgtacaacaggtaaattaccagttccatggccaacattagttacaacatttggttatggtttacaatgttttgcacgttatccagatcacatgaaacaacatgatttttttaaatcagcaatgcctgaaggttatgttcaagaacgtacaattttttttaaagatgatggcaactataaaacacgtgctgaagttaaatttgaaggtgatacattagttaatcgtattgaattaaaaggtattgattttaaagaagatggtaatattttaggtcataaattagaatataattataactcacataatgtttatatcatggcagataaacaaaaaaatggtattaaagttaattttaaaattcgtcataatattgaagatggttcagttcaattagcagatcattatcaacaaaatacacctattggtgatggtccagttttattaccagataatcattatttatcatatcaatcagcattatcaaaagatccaaatgaaaaacgtgatcacatggttttattagaatttgttacagcagcaggtattacattaggtatggatgaattatataaataatcgaaaaaaaaaaccccgcttcggcggggttttttttttgttccgg 3'

## Level 2 multi-TU constructs

Level 2 plasmid backbone: pTU2-a-RFP

### Level 2 construct variant 1

Sub-units: Transcription unit 1 variant 1, Transcription unit 2 variant 1,

Sequence (Excluding plasmid backbone): 5' atctctatttgacagctagctcagtcctaggtataatgctagcgtactcacacaggaaacccatatggtttcaaaaggtgaagaattatttacaggtgttgttccaattttggttgaattagatggtgatgttaatggtcataaattttcagtttcaggtgaaggtgaaggtgatgcaacttatggtaaattaacattaaaatttatttgtacaacaggtaaattaccagttccatggccaacattagttacaacatttggttatggtttacaatgttttgcacgttatccagatcacatgaaacaacatgatttttttaaatcagcaatgcctgaaggttatgttcaagaacgtacaattttttttaaagatgatggcaactataaaacacgtgctgaagttaaatttgaaggtgatacattagttaatcgtattgaattaaaaggtattgattttaaagaagatggtaatattttaggtcataaattagaatataattataactcacataatgtttatatcatggcagataaacaaaaaaatggtattaaagttaattttaaaattcgtcataatattgaagatggttcagttcaattagcagatcattatcaacaaaatacacctattggtgatggtccagttttattaccagataatcattatttatcatatcaatcagcattatcaaaagatccaaatgaaaaacgtgatcacatggttttattagaatttgttacagcagcaggtattacattaggtatggatgaattatataaataatcgaaaaaaaaaaccccgcttcggcggggttttttttttgtttgcctgccctatttgacagctagctcagtcctaggtataatgctagcgtactcacacaggaaacccatatggtttcaaaaggtgaagaattatttacaggtgttgttccaattttggttgaattagatggtgatgttaatggtcataaattttcagtttcaggtgaaggtgaaggtgatgcaacttatggtaaattaacattaaaatttatttgtacaacaggtaaattaccagttccatggccaacattagttacaacatttggttatggtttacaatgttttgcacgttatccagatcacatgaaacaacatgatttttttaaatcagcaatgcctgaaggttatgttcaagaacgtacaattttttttaaagatgatggcaactataaaacacgtgctgaagttaaatttgaaggtgatacattagttaatcgtattgaattaaaaggtattgattttaaagaagatggtaatattttaggtcataaattagaatataattataactcacataatgtttatatcatggcagataaacaaaaaaatggtattaaagttaattttaaaattcgtcataatattgaagatggttcagttcaattagcagatcattatcaacaaaatacacctattggtgatggtccagttttattaccagataatcattatttatcatatcaatcagcattatcaaaagatccaaatgaaaaacgtgatcacatggttttattagaatttgttacagcagcaggtattacattaggtatggatgaattatataaataatcgaaaaaaaaaaccccgcttcggcggggttttttttttgttccgg 3'